HOT WORK CHECKLIST / PERMIT

Alppila

Vessel:

Forecastle area

Location/Workplace/Item No.:

Drydock

Alarm Address: PROFILGATAN 23, 261 35 LANDSKRONA, Dock / Quay:

Work method: 	☒ Welding	☒ Grinding	☒Cutting	☐Soldering	☐Hot air           ☐ Other :

2026,08,13

Permit is valid from and to (maximum one day or one shift)

Date

19,00

0

08,00

Start Time                                                                                       Stop Time:

Can the combination of Method, Material & Environment (MME method) cause a fire hazard?	Yes ☒	No ☐

| § | No | Item |  |
| --- | --- | --- | --- |
| ☒ |  | 0 | The permit issuer is appointed. |
| ☒ |  | 1 | The person who will carry out the work is qualified to perform Hot Work with valid Swedish Hot Work Certificate. |
| ☒ | ☐ | 2A | Click or tap here to enter text. A competent Fire-Watch has been arranged for monitoring the hot work and has a valid Swedish Hot Work Certificate. If not, please specify the motivation in the box below (for further information see page 2 item fire-watcher 1.): |
| ☒ |  | 2B | Post-work monitoring (≥ 1 hour) has been arranged through the competent fire-watch. |
| ☒ |  | 3 | A permit for working in tank, space, pipeline that contain or have contained a flammable product have been stated (“Atmospheric test in confined spaces”) |
| ☒ |  | 4 | The workplace is tidy and if necessary wetted down. |
| ☒ |  | 5 | Combustible material in and near the workplace is removed, protected by covering and/or screened off. |
| ☒ | ☐ | 6A | There are heat-conducting constructions and concealed combustible structural elements. |
| ☒ |  | 6B | These are protected and accessible for immediately extinguishing fire. |
| ☒ | ☐ | 7A | There are gaps, holes, penetrations and other openings in and near the workplace. |
| ☐ |  | 7B | These openings are sealed or checked and protected. |
| ☒ |  | 8 | Approved, working and sufficient fire-fighting equipment of the correct type is available to immediately extinguish fire. |
| ☒ |  | 9 | Welding equipment is free from defects. |
| ☒ |  | 10 | The emergency services/fire brigade can be alerted immediately |

Signature Crew

Automatic fire alarm & Automatic fire extinguishing system are disconnected during the work: Yes☒   No☐   N/A☐

Disconnected area:

Confirmation that all safety regulations have been reviewed:

| Role | Name / Signature | Company | Comment |
| --- | --- | --- | --- |
| Vessel Rep. |  |  | Aware of work & position, fire alarm, fire system disconnection. |
| Permit Issuer Work Start | - Chiplakovski | ODD | Work can start - Checklist completed, Hot Work certificates reviewed and approved |
| Fire Watch 1 |  |  |  |
| Fire Watch 2 |  |  |  |
| Hot Work Operator 1 |  |  |  |
| Hot Work Operator 2 |  |  |  |
| Hot Work Operator 3 |  |  |  |
| Permit Issuer Work Completed |  |  | Work is completed, post-work monitoring performed and fire safety at the workplace has been checked. |
| Vessel Rep. Work Completed |  |  | Work is completed, post-work monitoring performed and fire safety at the workplace has been checked. Fire alarm, fire system reactivated. |

Short List - Responsibilities and Guideline – Hot Work

# Detailed description is documented in the management system. This short list is to be considered as a reminder for the Hot Work Team to enable a secure and efficient hot work.

Permit Issuer

- Owner of the Hot Work and issues a permit for the Hot Work according to the Company checklist

- Verify that all people in the hot work team have valid Nordic hot work certificates

- Supervision of Hot Work and ensure the work is performed according to the Company rules.

Fire-watcher

- The Fire Watch is mandatory for all hot work inside vessels. Work on the exterior of the vessel or in the docks may be conducted without a fire watch. Extra Risk evaluation must be carried out before any decision to do a hot work without a fire watch. The outcome of the Risk evaluation must be specified in the box under 2 A on page 1. Note: If any changes in the hot work scope during the validity of the permit a new risk evaluation and permit must be done.

- Shall have a visual identification/marking that the person is a Fire Watch (fire watch west)

- Must remain at the hot work location until there is no risk of fire, minimum 1 hour after work completed.

- The fire watch must always be present at the location of the hot work, including breaks.

Raising the alarm onboard vessel

- Activate fire alarm onboard

- Dial 112 and state the location (SHIPYARD LANDSKRONA, PROFILGATAN 23, LANDSRKONA)

- Inform ODD project manager, foreman and Vessel crew.

- Meet the fire brigade at the gate and let them in to the yard

- Guide/Follow the fire brigade to the exact location of the fire

Work Completed

- Must always be signed off by Permit Issuer and the Vessel Representative.

- Signed Checklist should be scanned and stored at the Project server on foto-server by Permit Issuer

General safety

- At least 2 x 6 kg certified portable Powder fire extinguishers close to the Hot work area. Supplementary extinguishers can be added if needed.

- At end of day/shift, gas hose/cables should be disconnected from the gas central

- Cable/hose protection should always be used in areas where heavy vehicles are operating

- Permit issuer is not allowed to be fire guard or perform the actual hot work

- The person performing the actual hot work is not allowed to be fire guard.

- All deviations should be reported to HSEQ